Fathom One ROV Assembly Instructions

10/31/17

CPU ASSEMBLY

Materials Required

- 4x #4-40 0.25’’ nylon screws

- 4x #4-40 0.25’’ nylon standoffs

- 4x #4-40 nylon hex nuts

- 1x CPU mount

- 1x Raspberry Pi CPU

- 1x Fathom Robotics Shield

- 1x CSI cable

- 2x 10K ohm NTC Thermistor w/ wire leads

- 1x 16GB SD card

Tools Required

- Phillips screwdriver

- Flat-nosed pliers

- Small flat head screwdriver

Assembly Instructions

- Insert flashed SD card into Raspberry Pi CPU

- Insert CPU mounting screws into Raspberry Pi.

- Insert .25’’ standoffs under Pi

- Place mounting plate onto screws

- Secure mounting plate with 4 hex nuts

- Install camera ribbon cable

- Use a small amount of glue to secure ribbon cable into receptacle

- Install robotics shield using GPIO pins

- Install thermistors into robotics shield

HULL ASSEMBLY PT 1

Materials Required

- 9x #6-32 0.75’’ stainless hex head screw

- 9x #6-32 0.25’’ aluminum threaded standoff

- 9x 20A custom ESCs

- 1x 150 PSI pressure transducer

- LocTite

Tools Required

- Socket wrench

- Custom pressure transducer mounting tool

Assembly Instructions

- Insert a #6-32 hex screw through ESC ring terminal

- Use a socket wrench to push the screw through the hull screw holes

- Tighten a .25’’ standoff onto the screw on the outside of the hull, use a small amount of Loctite between the hull and standoff to secure

- Repeat for all ESCs

- Mount depth sensor by using custom transducer wrench to screw into plastic mounting boss

RIGHT SIDE ESC WIRING

LEFT SIDE ESC WIRING

REAR ESC WIRING

HULL ASSEMBLY PT 2

Materials Required

- 1x female tether interface

- 1x locking nut

Tools

- Needle-nosed pliers

Assembly Instructions

- Thread cable assembly through top tether mounting hole

- Thread locking nut over the Ethernet plug, and screw onto tether interface

FRONT CAP ASSEMBLY

Materials Required

- 6x #1 1/8’’ self threading screws

- 4x #6 1/4’’ self threading screws

- Solder

- 15’’ 26 AWG

- 2x 3W star-type LEDs

- 1x Raspberry Pi camera module

Tools Required

- Phillips screwdriver

- Soldering iron

- Wire stripper

Assembly Instructions

- Secure camera onto front mounting plate using 2x #1 1/8’’ self threaded screws

- Solder 2x 3W LEDs together

- Mount LEDs onto front plate using 4x #1 1/8’’ self threading screws

- Screw mounting plate into the front cap using 4x #6 1/4’’ self threading screws

THRUSTER ASSEMBLY

Materials Required

- 12x #1 0.25’’ self threading screws

- x3 M3 stainless steel washers

- x3 M3 stainless steel locking nuts

- x3 custom 800 KV brushless motors

- x2 L/T propellers

- x1 R propeller

- x3 thruster housing

Tools Required

- Small phillips screwdriver

- Small allen wrench

- M3 socket wrench

Assembly Instructions

- Screw thruster mounting plate into housing using 4x #1 0.25 self threading screws

- Place propeller over motor bell, and install a washer and lock nut to hold it in place

- Place motor bell into mounting plate and lock using 2x set screws

HULL ASSEMBLY PT 3

Materials Required

- 1x charging interface assembly

- 1x locking nut

- 1x 3S balance lead extension

Tools Required

- Needle-nosed pliers

Assembly Instructions

- Clip the 3S balance lead extension close to the female plug

- Solder male-side 3S balance lead extension to female charging plug

- Thread 3S balance lead and plug assembly through hull charging port

- Thread the locking nut over the charging cables, and tighten into place

HULL ASSEMBLY PT 4

*QUALITY RISK AREA*

Materials Required

- 1x CPU assembly

- 1x front cap assembly

- 1x hull assembly

- 1x 5200mAh LiPo battery pack

- 1x steel ballast (2.85’’ x 2.5’’ x 0.25’’)

- 1x steel ballast (3.64’’ x 2.5’’ x 0.25’’)

Tools Required

- 1x small flat head screwdriver

- 1x test tether

- 1x test router

- 1x test smart device

Assembly Instructions

- Connect the Ethernet plug to the Raspberry Pi CPU, and the lights, transducer, ESC signal wires, PoE wires, and MOSFET switching leads to the Robotics Shield.

- Slide shield/Pi assembly into its mounting slot

- Connect the ESC power lines and the CSI cable to the Raspberry Pi camera module

- Connect battery to perform diagnostics

- Seal off the Raspberry Pi using hot glue between the Pi and the robotics shield

- Install 506g of steel ballast into ballast mounting brackets

HULL ASSEMBLY PT 5

*QUALITY RISK AREA*

Materials Required

- 624/625 potting foam

- Mixing cup

- Mixing stick

Tools Required

- Potting fixture

Assembly Instructions

- Mix 2.1oz of each side of the foam potting material into a mixing container, and mix thoroughly.

- Pour the foam mixture into the tail of the drone through the bottom of the hull

- Hold the battery in place to ensure the foam doesn’t push it forward into the allotted space for the front cap

- Let foam set

- Visually inspect to ensure there are no gaps between the foam and thruster interfaces

HULL ASSEMBLY PT 6

Materials Required

- 1x front cap o-ring

- Silicone grease

- 6x #6 5/8'' high-low self threading screw

- 1x hull/front cap assembly

- 1x silica packet

- Adhesive

Tools Required

- Phillips screwdriver

Assembly Instructions

- Place silica packet inside drone hull

- Use a small amount of adhesive to tack both thermistors onto the left and right sides of the battery

- Lubricate the front cap o-ring using silicone grease

- Place o-ring into the front cap open groove

- Screw the front cap into place using 6x #6 5 / 8’’ self threading screws

HULL ASSEMBLY PT 7

Materials Required

- Polycarbonate industrial adhesive

- 1x picatinny rail

Tools Required

- Rail mounting fixture

Assembly Instructions

- Place picatinny rail fixture over the drone body

- Apply adhesive to the picatinny rail piece

- Firmly press picatinny rail into its corresponding location and let set

THRUSTER ATTACHMENT

Materials Required

- x9 #6-32 plastic thumb nuts

- 1x finished hull assembly

- 3x finished thruster assemblies (2x left handed prop, 1x right handed prop)

Tools Required

N/A

Assembly Instructions

- Place motor leads over their corresponding thruster mounting posts according to the schematic below

- Slide thruster housing over thruster mounting posts

- Secure thruster housing using 3x plastic thumb screws per thruster

LEFT MOTOR WIRING

REAR MOTOR WIRING

RIGHT MOTOR WIRING

TOPSIDE ASSEMBLY

Materials Required

- 3x #6 1/4’’ self threading screws

- 4x #6 5/16’’ self threading screws

- 1x wireless transmitter top

- 1x wireless transmitter bottom

- 1x Hornet Routerboard

- 1x lock washer

- 1x antenna hex nut

- 1x 5dbi antenna

- 1x IPEX-RSMA(F)1.13-10

- 1x female tether interface

- 1x tether locking nut

Tools Required

- Phillips screwdriver

- Needle-nosed pliers

Assembly Instructions

- Place PCBA routerboard into bottom clamshell piece

- Secure using 3x #6 ¼’’ mounting screws

- Thread antenna port through top hole

- Secure using a locking washer and nut

- Thread female tether cable assembly through bottom hole

- Thread locking nut over Ethernet jack and screw into place

- Connect Ethernet jack to routerboard

- Install antenna

- Secure top clamshell piece using 4x #6 5/16’’ mounting screws